Laboratorijske vezbe sa Aurix TC375 – plan (2025/26)

# Teme (sumarno)

|  | Tema | Task (Prioritet) | FreeRTOS Mehanizam | Opis i uloga mehanizma | Zadatak |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | CAN Bus (Rx/Tx) | Bez FreeRTOS | - | CAN komunikacija: CAN loopback između 2 noda - unutar jednog kontrolera, a posle između 2 kartice. Definisani CAN ISR Tx, Rx, prijem, slanje. | - Preraditi originalni CAN loopback primer, tako da se ostvari CAN komunikacija između 2 kartice. - Osciloskopom analizirati CAN bus signale |
| 2. Copy Zadatak... | FreeRTOS_Basic | task_app_led1 (2) task_app_led2 (3) | - Periodičan task - Binary semaphore: za sinhronizaciju taska i ISR-a | - task_app_led1: periodičan, toggle-uje LED1 - task_app_led2: sinhronizacija taska sa prekidom (eksternim ISR aktiviran tasterom) upotrebom binarnog semafora (vTaskNotifyGiveFromISR() + ulTaskNotifyTake()). Nakon dobijene notifikacije, task toggle-uje LED2 | Task_app_led1 treba da ažurira novo kreiranu strukturu vremena SystemTime (hh:mm:ss) |
| 3. | System init | task_system_init (1) (file: App_SystemInit.c) | Task Management | Task za Inicijalizaciju sistema, priprema za kasniju konfiguracija svih periferija (CAN, I2C + RTC, SPI + EEPROM, UART...). Dati task kraju kreira sve ostale taskove i zatim briše sam sebe. | - U originalnom projektu FreeRTOS_Basic napraviti novi task za inicijalizaciju sistema (videti predlog strukture taska ispod ove tabele). - Testirati: nepromenjene funkcionalnosti iz FreeRTOS_Basic |
| 4. Copy Zadatak… | CAN Bus (Rx/Tx) | task_can_rx (4) task_can_tx (5) | Queue (za Rx) (sinhronizacija taska i ISR-a upotrebom queue) Periodican task (za Tx) | Upotreba redova za asinhroni prijem: CAN Rx: ISR stavlja primljeni podatak u queue, čime se aktivira odgovarajući task za parsiranje. Parsiranje primljenog podatka u SystemTimeReceived. CANTx: ciklicno slanje u postojećem tasku (task_app_led1) | Dodati: - inicijalizaciju za CAN u task_system_init - queue (po primeru iz knjige): xQueueCreate(), xQueueSend() - CAN Rx ISR, xQueueReceive()) – Task_CANRx |
| x. | UART Terminal | task_uart_rx (6) task_uart_tx (7) | Queue (za Rx) Queue (za Tx) | Početi od primera: iLLD_TC375_ADS_printf_scanf ASCLIN_Shell_UART_1_KIT_TC375_LK - RX: ISR + binary semaphore + ring buffer - TX: queue + dedicated UART TX task | 1.) Prouciti i testirati originalni primer iLLD_TC375_ADS_printf_scanf. (dodatni, slozeniji Debug Console primer: ASCLIN_Shell_UART_1_KIT_TC375_LK-neobavezno) 2.) Osciloskopom analizirati UART signale 3.) Ugraditi primer iLLD_TC375_ADS_printf_scanf u FreeRtos projekat, sa queue Rx i sa Tx opcijama: --- Mutex – rejected? --- queue Videti “Dodatak 2: UART -> FreeRtos predlog” |
| x. | OneEye Monitoring | Svi Taskovi | DAP Interface | Alat za vizuelizaciju varijabli (vreme, CAN brojači) u realnom vremenu putem DAP interfejsa bez uticaja na scheduler. | TBD |
| x. | I2C - RTC 3 Click | Task 1 (3) | Binary Semaphore | Task 1 inicira sekvencijalno čitanje 7 bajtova sa DS1307 i blokira se na semaforu dok I2C ISR ne završi prenos. | Upravljanje višebajtnim prenosom i pretvaranje blokirajućih poziva u neblokirajuće. |
| x. | SPI – EEPROM 5 Click | Task 4 (1) | Mutex + vTaskDelay | Task 4 piše podatke. Koristi Mutex za QSPI bus, a unutar petlje proverava WIP bit memorije uz vTaskDelay da ne troši CPU. | Rešavanje problema sporosti periferije (EEPROM write cycle) unutar RTOS-a. |
| x. | LIN Bus | Task 6 (2) | Software Timer | Task 6 koristi softverski tajmer za slanje LIN Master zahteva i čitanje Slave odgovora u definisanim vremenskim slotovima. |  |

### Dodatak 1: task_system_init

int main(void)

{

/* Low-level HW init (clock, MPU, etc.) */

hw_init_minimal();

/* Create system init task */

xTaskCreate(

task_system_init,

"SYS_INIT",

1024,

NULL,

tskIDLE_PRIORITY + 3,

NULL

);

/* Start scheduler */

vTaskStartScheduler();

/* Should never get here */

for (;;);

}

-------------------------------------------------------------------------------------------

void task_system_init(void *arg)

{

/* Full system init */

init_drivers();

// init_can();     			// will be done later

/* Create application tasks */

xTaskCreate(task_app_led1, "LED2", 512, NULL, 1, NULL);

xTaskCreate(task_app_led2, "COMM", 1024, NULL, 2, NULL);

xTaskCreate(task_canrx_handler, "COMM", 1024, NULL, 2, NULL);

/* Optional: signal "system ready" event here */

/* Delete self */

vTaskDelete(NULL);

}

### Dodatak 2: UART -> FreeRtos predlog

#include "FreeRTOS.h"

#include "task.h"

#include "semphr.h"

#include "queue.h"

static SemaphoreHandle_t g_uartRxSem;

static QueueHandle_t     g_uartTxQueue;

static uint8_t           g_rxBuf[128];

static volatile uint16_t g_rxHead, g_rxTail;

static void uart_rx_buffer_push(uint8_t ch)

{

uint16_t next = (g_rxHead + 1u) % sizeof(g_rxBuf);

if (next != g_rxTail)

{

g_rxBuf[g_rxHead] = ch;

g_rxHead = next;

}

}

static uint8_t uart_rx_buffer_pop(void)

{

uint8_t ch = 0;

if (g_rxHead != g_rxTail)

{

ch = g_rxBuf[g_rxTail];

g_rxTail = (g_rxTail + 1u) % sizeof(g_rxBuf);

}

return ch;

}

void UART0_RxISR(void)

{

BaseType_t xHigherPriorityTaskWoken = pdFALSE;

uint8_t ch;

/* Read one byte from ASCLIN RX FIFO */

ch = IfxAsclin_readRxData(MODULE_ASCLIN0);

uart_rx_buffer_push(ch);

/* Notify RX task */

xSemaphoreGiveFromISR(g_uartRxSem, &xHigherPriorityTaskWoken);

portYIELD_FROM_ISR(xHigherPriorityTaskWoken);

}

void task_uart_rx(void *arg)

{

uint8_t ch;

for (;;)

{

xSemaphoreTake(g_uartRxSem, portMAX_DELAY);

while (g_rxHead != g_rxTail)

{

ch = uart_rx_buffer_pop();

/* ovde obrada prijavljenog podatka */

/* npr. ako je '0'..'9' ili 'A'..'Z' */

/* ili parse string */

if (ch == '\r')

{

/* kraj linije */

}

}

}

}

void task_uart_tx(void *arg)

{

uint8_t ch;

for (;;)

{

xQueueReceive(g_uartTxQueue, &ch, portMAX_DELAY);

while (IfxAsclin_getTxFifoFillLevelFlagStatus(MODULE_ASCLIN0) != TRUE)

{

/* wait until TX FIFO has room */

}

IfxAsclin_clearTxFifoFillLevelFlag(MODULE_ASCLIN0);

IfxAsclin_writeTxData(MODULE_ASCLIN0, ch);

}

}

Slanje karaktera iz bilo kog task-a

void uart_send_char(uint8_t ch)

{

xQueueSend(g_uartTxQueue, &ch, portMAX_DELAY);

}

Detalji:

Prioriteti: 6 je najviši (najhitniji), 1 je najniži (pozadinski). - TBD

- Visok prioritet: Task za inicijalizaciju, Task  (UART RX) i Task 3 (CAN) – jer ne smeju da propuste podatke.

- Srednji prioritet: Task 1 (Timekeeper/I2C).

- Nizak prioritet: Task 4 (SPI/EEPROM) i Task 6.

Razno:

- I2C / SPI Callback: Pošto iLLD nema direktan callback, objasnite studentima da je ISR + Semaphore zapravo "RTOS način" implementacije callback-a.

- UART Mutex vs Queue:

- Za pisanje (printf): Uvek Mutex. To je najlakši način da terminal ostane čitljiv.

- Za prijem (scanf/cmd): Uvek Queue. To omogućava da sistem reaguje na komandu tek kada je ceo paket primljen, bez blokiranja ostalih taskova.

- EEPROM Pisanje: Poseban akcenat staviti na to da se Mutex ne sme držati predugo. Pravilan redosled:

- Uzmi Mutex -> Pošalji SPI komandu -> Čekaj SPI semafor (ISR) -> Proveri WIP bit (ako je zauzeto, vTaskDelay i probaj opet) -> Vrati Mutex.

- FreeRTOS pojasnjenje: Mutex vs Binary semphore

(obzirom da se koriste iste funkcije u oba slucaja (xSemaphoreTake, xSemaphoreGive))

Savet: Iako tehnički možete koristiti binarni semafor za zaštitu resursa, uvek koristite Muteks zbog zaštite od priority inversion problema. Za komunikaciju sa prekidima, uvek koristite Binarni semafor (ili još bolje, Task Notifications jer su brži).

- FreeRTOS STM32 primeri:

https://www.youtube.com/playlist?list=PLnMKNibPkDnExrAsDpjjF1PsvtoAIBquX

https://www.youtube.com/playlist?list=PLfIJKC1ud8gj1t2y36sabPT4YcKzmN_5D

# Zadaci (student)

Zuto: obrisati posle ili dipomac da sve osvezi

Primeri su bazirani na Infineonovoj ILL biblioteci i FreeRTOS-u (vise taskova, semafori, muteksi, redovi, itd.). Prilikom upoznavanje sa temom, prvo pogledati ReadMe.md fajl svakog primera. Posle upoznavanja sa originalnim projektom, neohodno je realizovati sledece:

- iLLD_TC375_ADS_FreeRTOS_Basic

Opis (originala): 1 taster + 2 LED. Periodični Task_app_led1 toggle-uje LED1, a tester eksternim interrupt-om putem event-a notifikuje Task_app_led2, koji toggluje LED2

Zadatak:

- dodati u periodic task strukturu za vreme (sec, min, hours) i potvrditi ponašanje u watch window

- C AN

Opis (originala): CAN loopback primer

Zadatak:

- original: postojeci Loopback primer prosiriti na 2 kartice

- ubaciti implementaciju u FreeRTOS primer

  - kreirati task za inicijalizaciju (gde ce se posle dodati inicijalizacija i za ostale periferije)

  - Rx: upotreba redova

  - Tx: mozda ne muteks, vec periodicno

- UART

printf/scanf, ubacen u FreRTOS

prijem: 3 queue ?

slanje: mutex ?

- OneOye

varijanta preko ADS-a, a ne preko UART-a, ubacen u FreRTOS

- I2C – RTC

Opis (originala): TBD

Zadatak:

- write/read (set alarm -> eksterni isr ?)

- ubaciti u FreeRTOS primer: write u init-u, periodican read

- SPI – EEPROM

Opis (originala): TBD

Zadatak:

- write/read

- ubaciti u FreeRTOS primer: write po zahtevu, periodican read

- LIN, ubacen u FreRTOS

- TOM, ubacen u FreRTOS

- Percepio tracer (full functionality, with free academic license)

https://freertos.org/Documentation/03-Libraries/02-FreeRTOS-plus/05-FreeRTOS_plus_Trace/RTOS_Trace_Instructions

# Teme sa detaljima

## Tema: CAN – obsolite, to be updated/abandoned …

Generalno, potrebno je proci kroz ceo .docx (pisan za TC275) i updajtovati ga sa TC375 kontrolerom i ovogodisnjim zadacima.

Posle upoznavanja sa gore navedenim, neophodno je:

- ostvariti CAN komunikaciju između 2 Lite kit-a (razmena uint32 varijable, dvosmerno).

Koraci rešavanja zadatka:

- detaljno proučiti CAN periferiju kontrolera!

- detaljno proučiti postojeći CAN loop back primer (= 1_MCMCAN1_1_KIT_TC375_LK_original)!

- realizovati CAN komunikaciju između 2 kit-a, preradom postojećeg CAN loop back primera.

Detalji: slati jednu uint32 varijablu koja se inkrementuje u svakom ciklusu slanja (Counter).

Potvrda realizovanog zadatka: poslati podaci sa jednog Lite kit-a se vidi na drugom kit-u i obratno.

(= 2_MULTICAN_1_TC275_2controllers)

- Navedene funkcije razmene podataka putem CAN-a dodati na postojeći FreeRTOS projekat (= 3_iLLD_TC375_ADS_FreeRTOS_Basic).

Detalji: slati jednu uint32 varijablu koja se inkrementuje u svakom ciklusu slanja (Counter), kao i podatke koji reprezentuju sate/minute/sekunde (struct, velicine 4 byta). Dakle sve staje u 8 byte-ova, što je jedna CAN poruka.

Potvrda realizovanog zadatka: poslati podaci sa jednog Lite kit-a se vidi na drugom kit-u i obratno.

Koraci rešavanja zadatka:

- Jedan Task (Tx): realizovati slanje iz periodičnog Task-a (CAN_Tx_Task, npr. 100 ms), a prijem u CAN Rx Interrupt-u.

Generalno, postoje 2 CAN interrupta: Rx i Tx. Rx – za prijem, Tx – za konfirmaciju slanja, koji se u ovom delu zadatka ne koristi.

- Dva Taska (Tx / Rx): realizovati slanje iz periodičnog Task-a (“CAN_Tx_Task”, npr. 100 ms, a prijem u Extended Tasku – “CAN_Rx_Task”).

Detalji: u CAN Rx Interrupt-u se sadržaj CAN mailbox-a odmah kopira u globalnu varijablu i setuje Event za task (CAN_Rx_Task). Posledično, FreeRTOS će odblokirati taj Task, u kome će se nastaviti dalja obrada podataka (npr. izdvajanje signala iz CAN poruke). Videti code snipet dole.

(Ranije = 3_MULTICAN_1_TC275_2controllers_with_freeRTOS)

- Queue: Kao pod b), ali se odblokiravanje CAN_Rx_Task kao i slanje primljenog podatka iz RX interrupta ka datom RX tasku vrši putem mehanizma redova u freeRTOS-u (queue).

- Dva Taska (Tx / Rx) + TxConfirmation: Realizovati slanje podatka na CAN bus tek kada se dobije potvrda je prethodni podatak poslat (to je ujedno maksimalno brzo slanje podataka)

Detalji: zadatak se realizuje upotrebom Tx Confirmation mehanizma, tj. putem CAN TX interrupt-a, koji setuje Event za CAN_Tx_Task, iz koga se šalje podatak na CAN. Dakle, CAN_Tx_Task  se ne izvršava periodično, već se pozove transmitCanMessage() funkcija samo prvi put (npr. Flag-om iz main funkcije), dok se svako naredno slanje realizuje iz CAN_Tx_Task-a.

OVO NISAM SLAO STUDENTIMA:

- Tri Taska (2 Tx / 1 Rx) + TxConfirmation: Dodati još jedan CAN_Tx_Task koji šalje poruku na CAN (dakle imate 2 odvojena taska: CAN_Tx_Task i CAN_Tx_Task1). Regulisati upotrebu CAN periferije (= resursa) upotrebom:

- FreeRTOS mutexa ili

- Mastering-the-FreeRTOS-Real-Time-Kernel.v1.1.0_vidiUartAdc_str10.3.7.pdf)

- Realizovati WakeUp funkcionalnost (RX linija dovedena i na dodatni pin, konfigurisan kao eksterni interrupt)

- Realizovati (posmatrati) detekciju BusOff stanja (2 CAN zice spojene)

ISR to task event signalling (iLLD_TC375_ADS_FreeRTOS_Basic project):

IFX_INTERRUPT(SCUERU_Int0_Handler, 0, ISR_PRIORITY_SCUERU_INT0)

{

BaseType_t externalIsrTriggered;

/* Notify task from ISR */

vTaskNotifyGiveFromISR(g_AppTaskHandle, &externalIsrTriggered);

…

}

void task_app_led2(void *arg)

{

g_AppTaskHandle = xTaskGetCurrentTaskHandle();

app_init();

while (1)

{

// Wait for task notification and then toggle LED2

ulTaskNotifyTake(0, portMAX_DELAY);

…

}

}

- Integracija u jedan zajednički projekat (sa drugim grupama)

- Dokumentacija

CANoe?

## Tema: I2C – to be updated

- TBD

## Tema: SPI – to be updated

- Posle gore navedenog neophodno je ostvariti upis i čitanje sa EEPROM-a preko SPI-a  (EEPROM na kitu je povezan sa FT232) (upisati jedan bajt i posle ga pročitati)

- Navedene funkcije upisa / čitanja dodati na postojeći FreeRTOS projekat: upis samo jednom - iz Task-a, čitanje posle - iz drugog task-a

- Integracija u jedan zajednički projekat (sa drugim grupama)

- Dokumentacija

## Tema: LIN – to be updated

- Izučiti postojeći Infineon primer za LIN (jednosmerna komunikacija)

- Posle gore navedenog neophodno je ostvariti komunikaciju između 2 kit-a (8 byte-ova, dvosmerno), bez freeRTOS-a, slično kao kod CAN primera

- Navedene funkcije razmene podataka dodati na postojeći FreeRTOS projekat: slanje iz Task-a, prijem u Interrupt-u

- Integracija u jedan zajednički projekat (sa drugim grupama)

- Dokumentacija